___________________________________________

| Atari SLM804 Printer Emulator Version 1.2 |

|    Copyright (c) 1988 Atari Corporation   |

|___________________________________________|

EMULATOR DISK CONTENTS

The SLM804 Printer Emulator Version 1.2 program disk con-

tains  the  following  program  and  data  files.  If you

currently have Version 1.1, you need only  replace  files

labeled "mod" and install files labeled "new".

SLM804 Printer Emulator

AUTO       mod

1ST_PRNT.DOT

LIST06LS.FNT   mod

RAZB10LS.FNT   new   (hand-bolded Razor 10 point)

RAZI10LS.FNT   new   (hand-italicized Razor 10 point)

RAZR06LS.FNT   new   (Razor 6 point)

RAZR10LS.FNT   mod

README.DOC   new   (this document)

SDUMP.PRG

SETUP630.ACC   mod

SETUP630.PRG   mod

SETUP630.RSC

SLM804.HEX

SPEEDPRN.PRG   new   (fast output for text files)

TEST.PRG

TYPE10LS.FNT   mod

SERIAL SELF-TEST FIX

When in serial mode (print data source is external), Ver-

sion  1.1  would "bomb" if self-test was attempted.  Ver-

sion 1.2 has this problem fixed.

LARGE FONTS FIX

Large fonts (those whose graphics form exceeds 64K) would

not  bold or italicize correctly in Version 1.1.  This is

fixed in Version 1.2.   Font  files  may  now  be  up  to

200,000 bytes in length with a 128-pixel height as a max-

imum.

LARGE HMI FIX

When setting large HMI values (using  <esc><US>n),  back-

space would not use the correct spacing amount.  Only HMI

values less than 52 work correctly in Version 1.1.   Ver-

sion  1.2  correctly  backspaces with HMI up to the docu-

mented maximum of 125.

THICKNESS FIELD FIX

Fonts to be bolded will hang the Emulator  while  loading

if  the value in the thickness field (bold amount) in the

font header is inadvertently set to  zero.   Version  1.2

will use a default value of three if zero is found there.

PAGE BREAK FIX

The Emulator preserves the current print position  across

a  page  break  and no longer performs a logical carriage

return.

BIT-MAP MODE FIX

Documentation for the Emulator indicates that the  escape

sequence       for       bit-map      graphics      print

(<esc><DC2>G$hhhhhhhh<sp>)  may  optionally  include  the

dollar  sign  immediately preceding the hex digit string.

In Version 1.1, "$" will prematurely  exit  bit-map  mode

without  printing the bit-map.  Version 1.2 tolerates "$"

correctly.  To be on the safe side,  applications  should

omit the dollar sign.

Note:  the Emulator documentation  does  not  make  clear

what SCALE FACTOR is for bit-map printing and what values

may be used.  The only acceptable values are 1, 2,  or  4

and  they define the resolution (300, 150, or 75 dots per

inch  respectively)  in  which  the  bit-map  image  will

appear.   Destination  X  and  Y values are always in 300

dots per inch units, however.

SELF-TEST FORMAT ENHANCEMENT

Self-test displays all characters  of  all  fonts  loaded

providing that there is enough room on one page.  Version

1.1 determines this cut-off point (where there is no more

room  for  font  characters) based only on one paper size

(letter-size).  Version 1.2  adjusts this  cut-off  point

appropriate to any of the 4 paper sizes that can be found

in the paper cassette.

AUTO-BOOT ESCAPE ENHANCEMENT

Certain font files with inconsistent  header  information

(caused  by  file  system corruption, etc.) may crash the

system when the Emulator is loading fonts.  This  can  be

very  inconvenient if the Emulator is configured to auto-

boot on a hard disk.  Version 1.2 looks for  the  depres-

sion  of  the  left or right <shift> key before each font

file is loaded and exits  without  installing  itself  if

that  key  was depressed.  Make certain to wait until the

screen  lights-up  after  reset  before  depressing   the

<shift> key.

HARDWARE BLIT ENHANCEMENT

Version 1.1 uses a specially-designed software blit  rou-

tine  to  "build" the graphics images of text as the page

is being printed.  Version 1.2 will engage  the  hardware

blit  to  provide this function if the BLiTTER is present

in the system.  The advantage of using hardware blits  is

that  a  higher density of characters can be printed than

is possible with software blits.  Heavy use of overstrik-

ing, very small fonts packing the page with large numbers

of characters, etc. can cause  a  CHAR  DENSITY  OVERLOAD

error.   When this error occurs, the top part of the page

appears normal up to  a  certain  horizontal  line  below

which  appears  an  area  filled  with white space, black

space, or a repeating pattern.  The threshold where  this

error occurs is greatly extended when a BLiTTER is avail-

able to boost blit performance.

Note:  some BLiTTERs have been found to  cause  a  system

crash  when  used  simultaneously  with DMA as is done by

Version 1.2.  If the system crashes  when  printing  with

the Emulator, try disengaging the BLiTTER via the Desktop

Options Menu.

SPEED PRINT ENHANCEMENT

A higher maximum printing speed  is  now  possible  using

Version  1.2.   The  maximum  print speed for Version 1.1

appeared to be about five pages per minute.   Of  course,

this maximum was only possible if the application supply-

ing the print data was doing so  at  a  reasonably  rapid

rate.   Dumping  a  file  to the printer from the Desktop

would perform at this maximum rate.  However, Version 1.2

has  a  new  feature which provides maximum throughput of

about eight to nine pages per minute.

Note:  some restrictions apply to achieve this top speed.

All the data to be printed must be in RAM at the time the

SPEED PRINT <esc> code  is  issued.   Reverse  line-feed,

engaging  fonts  taller or shorter than the current font,

changing the VMI values, double-sided printing  and  some

other  Emulator  features  may have unpredictable results

during speed printing and should be avoided.

To invoke SPEED PRINT, send <esc> * to the  printer  with

the  a1  register  pointing to a structure in memory con-

taining the data to be printed.  The first long  word  of

this  structure  is a pointer to one byte beyond the last

character to be printed.  The print  data  itself  should

follow this end pointer.

a1 -->   end ptr (long) ---|

|

print data        |

<-------|

SPEEDPRN.PRG is a rudimentary program that reads  a  file

into RAM and SPEED PRINTs it using this new feature.

BIT-MAP PRINTING ENHANCEMENT

The bit-map printing by image number feature was added to

Version 1.2 since existing word-processors couldn't issue

the bit-map print command.  Also there  are  difficulties

for the mere mortal in using the bit-map print feature of

Version 1.1 that could be overcome with this new  feature

in Version 1.2.

The idea here is to load bit-map images from  files  into

memory,  hook  them up to the Emulator with a linked list

structure, and  terminate  &  stay  resident.   With  the

images  residing  in memory arranged in this linked-list,

they could be easily printed  by  number  with  a  simple

<esc> code sequence from a word processor.

A simple program is needed to load images,  connect  them

to  the  linked list of the Emulator and terminate & stay

resident.  The  program  should  form  the  same  20-byte

header for each image loaded as is used by the previously

documented BIT-MAP PRINT  <esc>  code.   Preceeding  this

header should be the link pointer which links to the next

image or 0 to indicate the last in the list.   The  first

(actually, the 0th) image is connected to the Emulator by

writing its address to an address  within  the  Emulator.

The  address  to  write to is found by taking the address

found in the trap #13 vector (at location $000000B4), and

subtracting  16  decimal  from  it.   If a long word 0 is

found there, no images have  previously  been  installed.

Otherwise,  the value is presumed to be an address of the

0th image header in the linked list of images.

---$B4

|

|

|    .dc.l  LINK POINTER (or 0) -->  NEXT LINK POINTER (or 0) ----

|    .dc.l  xx                       .dc.l   s_form (bit-map #0) |

|    .dc.l  xx                       .dc.w   s_xmin              |

|    .dc.l  xx                       .dc.w   s_ymin              |

-->  Emulator trap #13 intercept     .dc.w   s_nxln              |

<body of Emulator code>        .dc.w   b_width             |

.dc.w   b_height            |

.dc.w   d_xmin              |

.dc.w   d_ymin              |

.dc.w   scalefac            |

.dc.b   <image data>        |

|

|

NEXT LINK PTR (or 0 etc.) <-------

<bit-map #1 structure, as above>

Once the images are  installed  by  a  terminate  &  stay

resident  program,  they  may be invoked (printed) from a

word   processor   or   other   application   by    using

<esc><DC2>#n[,x,y,s]<sp>,  where  n  is  an  ASCII string

equivalent of  the  decimal  image  number.   This  image

number  corresponds  to  the  order in the linked list of

images starting with image #0 up to a  maximum  of  image

#32767.   Optionally,  a  parameter  list  may  follow to

specify a new X and Y position for the top-left corner of

the  bit-map  image  as well as a new scale factor "s" to

override the default values already installed.

These X and Y values are in laser pixel units of 300 dots

per  inch  and  the  scale factor may be 1, 2, or 4 only.

Any parameter (except image number) may be omitted if the

default value is desired by placing a null string between

commas.  For example, to print image #4 at the default  X

position   but   with   a  new  Y  value  of  1250,  send

<esc><DC2>#4,,1250<sp>.  Regardless  of  whether  or  not

this  optional  parameter  list  is  used, the <esc> code

sequence must be terminated by a space character.

______________________________________

| *** Atari SLM804 Printer Caveats *** |

|______________________________________|

(1)  If an SLM804 Laser Printer is connected to the ST  Hard

Disk  (DMA bus) port, then the printer must be powered-

on in order for  floppy  or  hard  disk  operations  to

occur.

(2)  For the best print results on an SLM804 Laser  Printer,

set  up  the application software to maintain a minimum

half-inch left and right page margin.